Application Note 12

Avoiding Evaporation

0.  Introduction

Depending  on  incubation  conditions,  small  volumes  of  cell  culture  medium  may

evaporate  quickly,  especially  during  long-term  experiments.  Furthermore,  all  cell

culture incubators need a fairly long time to recover humidity, particularly after door

openings.  While  temperature  and  CO2 is  recovered  within  minutes,  full  humidity

recovery can take hours.

The above  graph shows the  temperature and humidity  inside a CO2 incubator  over a 10  hour

day,  during and  after  briefly opening  the  door.  The graph  shows  only minor  differences  in

temperature  over  the   day,  i.e.,  between   36°C  and  37°C.     However,  it  shows  significant

differences in relative humidity over  the day, i.e., between less than 50 %  and the steady state

of 93  %. An  important fact to  note is  that this  incubator had not  been opened  all afternoon,

except briefly just before 5 o’clock.

Therefore, for controlling and decreasing evaporation we recommend the following

techniques:

1. A Petri dish with wet tissues (Page 2)

2. An Olaf humidifying chamber (Page 2)

3. Parafilm (Page 3)

4. Silicon oil (Page 3)

Application Note 12

© ibidi GmbH, Version 2.0, Elias Horn, August 25, 2011

Page 1 of 3

Application Note 12

1.  A Petri Dish with Wet Tissues

A simple humidifying chamber can be made using a

10 cm Petri dish with wet tissues inside (as shown

on    the    right).    Use   EDTA    (Ethylene-Diamine-

Tetraacetic Acid) stabilized water and sterile tissues

for longer incubation periods. One advantage of this

technique   is   that   it  allows      access   with   low

resolution  microscopy  to  check,  for  example,  the

adherence of cells. This technique is recommended

for use inside cell culture incubators.

2.  Olaf Humidifying Chamber

The  Olaf  Humidifying   Chamber  (Cat  No.  80008)   prevents  problems  caused  by

evaporation, especially during long term   assays. Olaf is a humidifying chamber that

can be completely sealed, which prevents loss of humidity while inside an incubator.

The Olaf chamber can be sterilized with a mixture of 70% ethanol and water, and is

recommended for use inside cell culture incubators.

Procedure:

    Fill a standard 6 well plate with sterile water (i.e., 4 ml per well) and place the

plate  into  an   Olaf  chamber.  Use  EDTA   stabilized  water  for  longer  term

experiments.

    Put a μ-Slide on top of the 6 well plate, close the Olaf chamber and place the

chamber into an incubator.

An Olaf Chamber with aμ-Slide  18 well inside. A water-filled 6 well plate provides humidity.

Another option is to place a  μ-Slide Click Rack (Cat No. 80007) onto the water-filled

plate. This option allows convenient handling of up to 4 μ-Slides in parallel.

Please  note,  however,  that  the μ-Slide  18  well  is  not  compatible  with  the μ-Slide

Click Rack. In this instance we recommend placing a  μ-Slide 18 well directly on top

of the 6 well plate.

Application Note 12

© ibidi GmbH, Version 2.0, Elias Horn, August 25, 2011

Page 2 of 3

Application Note 12

3.  Parafilm

Placing  small   strips  of   Parafilm  (produced   by  the   Pechiney  Plastic   Packaging

Company) onto the reservoirs of the slides will prevent the effects of evaporation.

Just fill the slide as recommended and then stretch the Parafilm on top until it fits

tightly.

The Parafilm technique is recommended for use with stage top incubators and with

non-humidified incubated microscopes.

4.  Silicon Oil

To prevent evaporation effects one can overlay the cell culture medium with Silicon

Oil AR 200 (available from Fluka, Sigma-Aldrich). Silicon oil is gas permeable and non-

toxic. Please don’t use mineral oil as this is harmful to the μ-Slides.

Procedure:

    Autoclave the silicone oil.

    Equilibrate oil and cell medium inside the incubator overnight. This step helps

in avoiding the formation of air bubbles and pre-warms all solutions to 37°C.

    Fill your slide with cells and medium.

    Overlay the medium’s surface with an appropriate amount of silicon oil. Do

not drip the oil directly onto the surface, rather let it run down the edges by

pressing  the  pipette  tip  onto  the  upper  side  of  the  reservoir.  For  example,

when using the μ-Slide VI      , fill each reservoir with 30 μl cell-free medium

0.4

and 30 μl silicone oil.

The  silicon  oil  technique  is  recommended  for  use  with  stage  top  incubators  and

when imaging for several hours using non-humidified incubated microscopes.

Silicon oil

Cell medium

μ-Slide VI

0.4

with silicon oil inside the reservoirs, which is used to decrease evaporation in non-

humid environments, e.g., on the microscope.

Application Note 12

© ibidi GmbH, Version 2.0, Elias Horn, August 25, 2011

Page 3 of 3